Supporting Information

Complete removal of heavy metals with simultaneous efficient treatment of etching terminal wastewater using scaled-up microbial electrolysis cells

Liping Huanga,*, Huilin Wana, Shiping Songa, Deqiang Liub, Gianluca Li Pumac,*

a Key Laboratory of Industrial Ecology and Environmental Engineering, Ministry of Education (MOE), School of Environmental Science and Technology, Dalian University of Technology, Dalian 116024, China

b Weihai Inspection and Research Institute of Products Quality, Standard and Metrology, Weihai 264209, China

c Environmental Nanocatalysis & Photoreaction Engineering, Department of Chemical Engineering, Loughborough University, Loughborough LE11 3TU, United Kingdom

Corresponding authors:

lipinghuang@dlut.edu.cn (L. Huang)

g.lipuma@lboro.ac.uk (G. Li-Puma)

Figures: 9

Tables: 4

Pages: 16

S2 Materials and methods

S2.1 Reactor construction

Figure S1 (A) Schematic isometric view of cylindrical single-chamber MEC reactors of (left) the 40 L (along with the appendixes), and (right) the 1 L. (B) Photos of the 1 L (above) and the 40 L (below) reactors.

S2.2 Bacterial community analysis

Electrodes samples were fragmented using sterile scissors, with cells extracted by rinsing three times with sterile water, and concentrated by centrifugation for 10 min at 10280 × g. The extracted DNA from sample was amplified using PCR reactions and universal 16S rRNA primers (Illumina Miseq) [1]. After the PCR amplifification, the amplicons were recovered using a gelose extraction kit (Cat: SK8131, Sangon Biotech (Shanghai) Co. Ltd., China) and rinsed with Tris-HCl solution. After amplification and quantification, the amplicons were sequenced using an Illumina Miseq following standard procedures. The data were optimized through removal of low-quality sequences, unrecognized reverse primers, and any ambiguous base calls, with a length < 200 bp. High quality sequences were aligned, grouped into OTUs (97% similarity) using the uclust algorithm, and a representative sequence from each OUT was classified phylogenetically assigned to a taxonomic identity (phylum, class, and genus level) using the RDP Naḯve Bayesian rRNA classifier at a confidence threshold of 90%. Rarefaction curves, Shannon and Simpson diversity indices, and species richness estimates of Chao1 were generated for each sample using QIIME software (v1.3.0). All analyses described were conducted in triplicates, and the means were reported.

S2.3 Calculation

where rH2 (mmol/d) and ΔGH2 (–237.1 kJ/mol) represent the H2 production rate and the molar Gibbs free energy for hydrogen combustion of the produced hydrogen in the MECs; ∆V is the difference of potential established between the counter and the working electrodes (V); I represents average current (A); 1/1000 (kW/W), 1000 (W/kW), 24 (h/d) and 3600 (s/h) are the transfer coefficients.

References

[1] L. Huang, F. Tian, Y. Pan, L. Shan, Y. Shi, B.E. Logan, Mutual benefits of acetate and mixed tungsten and molybdenum for their efficient removal in 40 L microbial electrolysis cells, Water Res. 162 (2019) 358−368.

[2] W. Teng, G. Liu, H. Luo, R. Zhang, Y. Xiang, Simultaneous sulfate and zinc removal from acid wastewater using an acidophilic and autotrophic biocathode, J. Hazard. Mater. 304 (2016) 159−165.

[3] M. Mahmoud, P. Parameswaran, C.I. Torres, B.E. Rittmann, Relieving the fermentation inhibition enables high electron recovery from landfill leachate in a microbial electrolysis cell, RSC Adv. 6 (2016) 6658−6664.

[4] L.B. Jugnia, D. Manno, M. Hendry, B. Tartakovsky, Removal of heavy metals in a flow-through vertical microbial electrolysis cell, Can. J. Chem. Eng. 97 (2019) 2608−2616.

[5] L. Zhang, Z. Xu, Z. He, Selective recovery of lead and zinc through controlling cathodic potential in a bioelectrochemically-assisted electrodeposition system, J. Hazard. Mater. 386 (2020) 121941.

[6] R.D. Cusick, B. Bryan, D.S. Parker, M.D. Merrill, M. Mehanna, P.D. Kiely, G. Liu, B.E. Logan, Performance of a pilot-scale continuous flow microbial electrolysis cell fed winery wastewater, Appl. Microbiol. Biotechnol. 89 (2011) 2053−2063.

[7] E.S. Heidrich, J. Dolfing, K. Scott, S.R. Edwards, C. Jones, T.P. Curtis, Production of hydrogen from domestic wastewater in a pilot-scale microbial electrolysis cell, Appl. Microbiol. Biotechnol. 97 (2013) 6979−6989.

[8] R. Moreno, M.I. San-Martín, A. Escapa, A. Moran, Domestic wastewater treatment in parallel with methane production in a microbial electrolysis cell, Renew. Energy 93 (2016) 442−448.

[9] L. Gil-Carrera, A. Escapa, P. Mehta, G. Santoyo, S.R. Guiot, A. Moran, B. Tartakovsky, Microbial electrolysis cell scale-up for combined wastewater treatment and hydrogen production, Bioresour. Technol. 130 (2013) 584−591.

[10] A. Escapa, M.I. San-Martín, R. Mateos, A. Moran, Scaling-up of membraneless microbial electrolysis cells (MECs) for domestic wastewater treatment: Bottlenecks and limitations, Bioresour. Technol. 180 (2015) 72−78.

[11] P. Zamora, T. Georgieva, A. ter Heijne, T.H. Sleutels, A.W. Jeremiasse, M. Saakes, C.J. Buisman, P. Kuntke, Ammonia recovery from urine in a scaled-up microbial electrolysis cell, J. Power Sources 356 (2017) 491−499.

[12] E.S. Heidrich, S.R. Edwards, J. Dolfing, S.E. Cotterill, T.P. Curtis, Performance of a pilot scale microbial electrolysis cell fed on domestic wastewater at ambient temperatures for a 12 month period, Bioresour. Technol. 173 (2014) 87−95.

[13] J. Chen, W. Xu, X. Wu, J.Q. E, N. Lu, T. Wang, H. Zuo, System development and environmental performance analysis of a pilot scale microbial electrolysis cell for hydrogen production using urban wastewater, Energy Convers. Manage. 193 (2019) 52−63.

[14] Brown RK, Harnisch F, Wirth S, Wahlandt H, Dockhorn T, Dichtl N, Schroder U. Evaluating the effects of scaling up on the performance of bioelectrochemical systems using a technical scale microbial electrolysis cell. Bioresour Technol 163 (2014) 206−213.

[15] L. Gil-Carrera, A. Escapa, R. Moreno, A. Moran, Reduced energy consumption during low strength domestic wastewater treatment in a semi-pilot tubular microbial electrolysis cell, J. Environ. Manage. 122 (2013) 1−7.

[16] R. Shen, Z. Liu, Y. He, Y. Zhang, J. Lu, Z. Zhu, B. Si, C. Zhang, X. Xing, Microbial electrolysis cell to treat hydrothermal liquefied wastewater from cornstalk and recover hydrogen: Degradation of organic compounds and characterization of microbial community, Inter. J Hydrogen Energy 41 (2016) 4132−4142.

[17] D.D. Leicester, J.M. Amezaga, A. Moore, E.S. Heidrich, Optimising the hydraulic retention time in a pilot-scale microbial electrolysis cell to achieve high volumetric treatment rates using concentrated domestic wastewater, Molecules 25 (2020) 2945.

[18] S.E. Cotterill, J. Dolfing, C. Jones, T.P. Curtis, E.S. Heidrich, Low temperature domestic wastewater treatment in a microbial electrolysis cell with 1 m2 anodes: Towards system scale-up, Fuel Cells 17 (2017) 584−592.

[19] M.I. San-Martín, R. Mateos, B. Carracedo, A. Escapa, A. Moran, Pilot-scale bioelectrochemical system for simultaneous nitrogen and carbon removal in urban wastewater treatment plants, J. Biosci. Bioeng. 126 (2018) 758−763.

[20] D. Jiang, M. Curtis, E. Troop, K. Scheible, J. McGrath, B. Hu, S. Suib, D. Raymond, B. Li, A pilot-scale study on utilizing multi-anode/cathode microbial fuel cells (MAC MFCs) to enhance the power production in wastewater treatment, Inter. J. Hydrogen Energy 36 (2011) 876−884.

[21] M. Dhadwal, Y. Liu, B.R. Dhar, Coupling microbial electrolysis cell and activated carbon biofilter for source-separated greywater treatment, Processes 9 (2021) 281.

[22] M. Blatter, C. Furrer, C.P. Cachelin, F. Fischer, Phosphorus, chemical base and other renewables from wastewater with three 168-L microbial electrolysis cells and other unit operations, Chem. Eng. J. 390 (2020) 124502.

[23] L. Yang, H. Cheng, Y. Ding, S. Su, B. Wang, R. Zeng, H.M.A. Sharif, A. Wang, Enhanced treatment of coal gasification wastewater in a membraneless sleeve-type bioelectrochemical system, Bioelectrochemistry 129 (2019) 154−161.

A

B

C

D

Figure S2 HPLC-ESI-MS mass spectra (A and C) and chromatograms (B and D) at retention times of (A and B) 16.76 min or (C and D) 14.70 min for real etching terminal wastewater.

Figure S3 Electrode potential and circuital current of 1 L (A) or 40 L (B) reactors fed with nutrient or progressive increase of real etching terminal wastewater (shown as ratios of wastewater and nutrient as 1 : 1, 3 : 1, 7 : 1, 18 : 1, or full wastewater) as a function of acclimation time.

Figure S4 Comparison of luminescence inhibition rates for low and low strength influents of etching terminal wastewater before and after treated by 1 L or 40 L MECs under closed circuital (CCC) or open circuital (OCC) conditions.

Figure S5 Heavy metal removal (A), COD removal and H2 production (B), effluent pH and conductivity (C), and electrode potential and circuital current (D) in 1 L reactor fed with high strength etching terminal wastewater and at an HRT of 5 d.

Figure S6 SEM observation on the cathodes (A, C, E and G) and the anodes (B, D, F and H) of 1 L (A ‒ D) or 40 L (E ‒ H) MECs fed with low (A, B, E and F) or high (C, D, G and H) strengths etching terminal wastewater.

Figure S7 Rarefaction curves of bacterial communities on the bioanode and the biocathode of the 1 L or 40 L reactors fed with low or high strengths etching terminal wastewater. The OTUs were defined by a distance limit of 0.03.

Figure S8 Weighted Fast UniFrac (A) principle coordinates analysis and (B) cluster of the bacterial communities on the basis of phylogenetic lineages at phylum level that samples contain.

Figure S9 Comparison of relative abundance of bacterial reads retrieved from the cathodes and the anodes of 1 L or 40 L MECs fed with low or high strengths etching terminal wastewater (thus abbreviated as 1LCat, 1LAno, 1HCat, 1HAno, 40LCat, 40LAno, 40HCat and 40HAno, respectively) and classified at (A) the phylum level and (B) the class level distribution of the most dominant phylum of Proteobacteria. Phyla and classes that represent less than 1.0% of the total bacterial community composition were classified as “others”.

Table S1 Summary of literature about practical wastewater treatment using microbial electrolysis cell technologies

| Source of wastewater | Reactor architecture | Working volume (L) | Anode | Cathode | applied voltage (V, vs. SHE) | Circuital current (mAa or A/m3 b) | pH | Solution conductivity (mS/cm) | Hydraulic retention time (h) | Gas production (m3/m3/d) | Influent (mg/L) | Removal efficiency (%) | Coulombic efficiency (%) | Products | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Simulated acid wastewater | Dual- chamber | 0.04 (biocathode) | Graphite fiber | Graphite fiber | 0.7 | 10 ‒ 51b | 3.0 | --- | 48 (batch) | --- | Zn(II): 15 ‒ 40; SO42-: 200 | Zn(II): 25 ‒ 100%; Sulfate reduction: 2.3 ‒ 37 mg/L/d | 2.3 ‒ 16 | ZnS | [2] |
| Landfill leachate | Dual- chamber | 0.32 (bioanode) | Graphite rod | Graphite fiber | 0.05 | 0.76b | 7.6 | --- | 17.8 | --- | COD: 2594 | 2 | 1.8 | --- | [3] |
| Simulated mixed heavy metals | Single- chamber | 1.0 (bioanode; biocathode) | Granular carbon | Granular carbon | 1.4 ‒ 1.5 | 0.1 ‒ 0.2b | 7.0 | 2.0 | 4.2 ‒ 6.7 | --- | Al(III): 2.7; Cu(II): 0.6; Fe(II):11; Pb(II): 0.7; Zn(II): 27 | Al(III): 100; Cu(II): 100; Fe(II): 100; Zn(II): 87 ‒ 100 | --- | --- | [4] |
| Simulated smelting wastewater | Dual- chamber | 0.2 | Carbon brush | graphite rod | 0.55 ‒ 0.95 | 2.0a | 4.6 ‒ 5.1 | --- | 20 (batch) | --- | Pb(II): 115; Zn(II): 150; Cd(II) < 1; Cu(II) < 1 | Pb(II): 98.5 (0.75 V); Zn(II): 98.7 (1.2 V) | --- | Pb and its oxide; Zn and its oxide | [5] |
| Winery wastewater | Single- chamber | 910 (bioanode) | Graphite felt brush | Stainless steel mesh | 0.9 | 0.1 ‒ 0.3b | 6.4 | 0.40 ‒ 2.3 | 24 | H2: 0.28; CH4: 0.24 | 760 | 62 | --- | --- | [6] |
| Domestic wastewater | Dual- chamber | 88 | Carbon felt | Stainless steel wire wool | 0.6 ‒ 1.1 | 0.04 ‒ 0.27b | 7.0 | --- | 24 | H2: 0.015 | 0 ‒ 4500 | COD: 34 | 55 | --- | [7] |
| Domestic wastewater | Single- chamber | 3 | Carbon felt | Stainless steel | 1.0 |  | 7.0 ‒ 7.2 | 0.49 ‒ 0.50 | 4 ‒ 24 | CH4: 0.001 ‒ 0.006 | 65 ‒ 78 | 19 ‒ 50 | 70 | --- | [8] |
| Municipal wastewater | Single- chamber | 10 | Carbon felt | Carbon paper/Ni | 0.9 | 186.3a | 7.0 | --- | 10 | 1.0 ‒ 2.6 | 270 | 60 ‒ 76 | 23 | --- | [9] |
| Domestic wastewater | Single- chamber | 7 | Graphite felt | Ni-based electrode | 0.7 | 0.2b | 7.2 | 1.4 | 48 (batch) | H2: 0.017 | 476 | 74 | --- | --- | [10] |
|  |  |  |  |  |  |  |  |  | 7 - 42 | --- | 70 ‒ 400 | 69 | --- |  |  |
| Urine wastewater | Dual- chamber | 5 | Titanium plates | Titanium plates | 0.5 ‒ 0.7 | 1.7b | 9.4 | 28.1 | 6 - 9 | --- | 4500 | 25 | 65 | --- | [11] |
| Domestic wastewater | Dual- chamber | 88 | Carbon felt | Stainless steel | 0.6 ‒ 1.1 | --- | 7.0 | 1.8 | 24 | H2: 0.07 | 147 ‒ 1976 | 30 - 60 | 41 | --- | [12] |
| Urban wastewater | Dual- chamber | 130 | Graphite felt | Stainless steelol | 1.0 | 0.27b | --- | --- | 48 | H2: 0.032 | 500 | 25 | 40 | --- | [13] |
| Domestic wastewater | Dual- chamber | 60 | Graphite plates | Stainless steel mesh | 0.4 | 0.72b | 6.5 | --- | 30 | --- | 636c | 67 | 11 | --- | [14] |
| Domestic wastewater | Single- chamber | 4 | Carbon felt | Carbon paper/Ni | 0.6 ‒ 1.0 | 0.04 ‒ 0.22b | 6.9 ‒ 7.4 | 0.6 | 4, 7 or 10 | H2: 0.05; CH4: 0.02 | 96 ‒ 135 | 70 | 9 ‒ 31 | --- | [15] |
| Cornstalk wastewater | Dual- chamber | 1.8 | Carbon nanotubes | Carbon fiber felt | 0.6 - 1.2 | --- | 7.0 | --- | 16 | H2: 0.004 | BOD/COD: 0.16 | 60 ‒ 80 | --- | --- | [16] |
| Domestic wastewater | Dual- chamber | 72 | Carbon felt | Stainless steel | 0.9 | 1.08 ‒ 1.36b | --- | --- | 0.36 ‒ 432 | H2: 0.016 | 4535 | 55 | 0 ‒ 131 | --- | [17] |
| Domestic wastewater | Single- chamber | 175 | Graphite felt | Stainless steel | 0.9 | 0.29b | 8.0 | 0.4 ‒ 0.8 | 5 | H2: 0.003 | 2594 | 64 | 28 | --- | [18] |
| Urban wastewater | Dual- chamber | 150 | Graphite felt | Graphite felt | 1.0 | 0.2b | 7.8 | 0.8 | 24 | --- | --- | 80 | 3 ‒ 10 | --- | [19] |
| Domestic wastewater | Single- chamber | 20 | Granular carbon | Carbon cloth /Pt/MnO2 | --- | 0.16 ‒ 0.2b | --- | ---- | 5, 10 or 20 | --- | 100 ‒ 1000 | 66 ‒ 80 | --- | 0.3 | [20] |
| Greywater | Dual- chamber | 0.4 (anode) | Carbon fiber | Stainless steel | 0.2 | 0.65b | 7.4 | 0.05 | 24 | --- | COD: 445 ‒ 485 | COD: 32 | --- | --- | [21] |
| Municipal wastewater | Dual- chamber | 168 (anode) | Reticulated vitreous carbon | Reticulated vitreous carbon | 2.5 | 0.06b | 7.3 ‒ 9.3 | --- | 4.5 ‒ 984 | --- | --- | 85 ‒ 95 | --- | Struvite; purified water | [22] |
| Coal gasification wastewater | Single- chamber | 1.4 | Carbon fiber | Stainless steel | 0.7 | 1.0 ‒ 1.2b | 7.0 | --- | 24 ‒ 48 | --- | 500 | 60 | 0.7 ‒ 1.5 | --- | [23] |

Table S2 CV data and EIS parameters obtained from Nyquist plots on the biotic or abiotic cathodes and anodes of 1 L or 40 L reactors fed with low or high strengths etching terminal wastewater (Eonset: reduction or oxidation onset potentials; Ep: reduction or oxidation peak potentials; Ip: reduction or oxidation peak currents; Rdif: diffusional resistance; Rs: solution and electrode resistance; Rct: charge transfer resistance).

| MEC | Influent |  | Cu |  |  |  | Ni |  |  |  | Zn |  |  |  | Organics oxidation |  |  |  | Rs (Ω) | Rct (Ω) | Rdif (Ω) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Eonset (V) | Ep (V) | Ip (mA) |  | Eonset (V) | Ep (V) | Ip (mA) |  | Eonset (V) | Ep (V) | Ip (mA) |  | Eonset (V) | Ep (V) | Ip (mA) |  |  |  |  |
| 1 L | Low strength | Cathode | 0.380 | ‒0.120 | ‒0.555 |  | ‒0.180 | ‒0.300 | ‒0.718 |  | ‒0.660 | ‒0.860 | ‒0.933 |  | 0.280 | 0.880 | 0.522 |  | 39.4 | 51.4 | 1685 |
|  |  | Anode | 0.340 | ‒0.040 | ‒0.509 |  | ‒0.200 | ‒0.260 | ‒0.637 |  | ‒0.680 | ‒0.880 | ‒0.880 |  | 0.300 | 0.520 | 0.580 |  | 35.2 | 60.2 | 1677 |
|  | High strength | Cathode | 0.404 | 0.164 | ‒1.832 |  | ‒0.056 | ‒0.226 | ‒2.013 |  | ‒0.636 | ‒0.806 | ‒2.056 |  | 0.224 | 0.434 | 2.177 |  | 34.2 | 30.0 | 1366 |
|  |  | Anode | 0.393 | 0.143 | ‒1.773 |  | ‒0.067 | ‒0.237 | ‒1.863 |  | ‒0.647 | ‒0.817 | ‒1.920 |  | 0.273 | 0.433 | 1.830 |  | 31.9 | 40.1 | 1453 |
|  | Low strength | Abiotic | 0.331 | 0.130 | ‒0.353 |  | ‒0.129 | ‒0.330 | ‒0.421 |  | ‒0.670 | ‒0.888 | ‒0.487 |  | 0.233 | 0.412 | 0.346 |  | 51.0 | 64.1 | 1726 |
|  | High strength |  | 0.350 | 0.110 | ‒1.496 |  | ‒0.080 | ‒0.260 | ‒1.795 |  | ‒0.660 | ‒0.860 | ‒1.821 |  | 0.280 | 0.410 | 1.623 |  | 43.7 | 55.4 | 1446 |
| 40 L | Low strength | Cathode | 0.360 | ‒0.020 | ‒0.558 |  | ‒0.160 | ‒0.280 | ‒0.563 |  | ‒0.660 | ‒0.840 | ‒0.616 |  | 0.360 | 0.620 | 0.379 |  | 51.7 | 57.6 | 1718 |
|  |  | Anode | 0.340 | ‒0.040 | ‒0.484 |  | ‒0.200 | ‒0.380 | ‒0.510 |  | ‒0.680 | ‒0.860 | ‒0.599 |  | 0.220 | 0.500 | 0.310 |  | 47.9 | 65.3 | 1722 |
|  | High strength | Cathode | 0.370 | ‒0.100 | ‒1.523 |  | ‒0.250 | ‒0.430 | ‒1.725 |  | ‒0.650 | ‒0.820 | ‒1.880 |  | 0.320 | 0.530 | 1.625 |  | 41.3 | 34.3 | 1425 |
|  |  | Anode | 0.360 | ‒0.160 | ‒1.256 |  | ‒0.240 | ‒0.380 | ‒1.038 |  | ‒0.670 | ‒0.850 | ‒1.180 |  | 0.290 | 0.550 | 1.180 |  | 38.5 | 39.4 | 1480 |
|  | Low strength | Abiotic | 0.331 | 0.130 | ‒0.353 |  | ‒0.129 | ‒0.330 | ‒0.421 |  | ‒0.670 | ‒0.888 | ‒0.487 |  | 0.233 | 0.412 | 0.346 |  | 62.1 | 67.2 | 1751 |
|  | High strength |  | 0.350 | 0.110 | ‒1.496 |  | ‒0.080 | ‒0.260 | ‒1.795 |  | ‒0.660 | ‒0.860 | ‒1.821 |  | 0.280 | 0.410 | 1.623 |  | 47.5 | 56.9 | 1470 |

Table S3 Comparison of binding energy and composite of different valences of heavy metals on the anodes and the cathodes of 1 L or 40 L reactors fed with high strength etching terminal wastewater

|  | Binding energy (eV) |  |  |  |  |  |  | Composite (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cu(0) 2p3/2 | Cu(II) 2p3/2 | Cu(II) 2p3/2,sat | Cu(0) 2p1/2 | Cu(II) 2p1/2 | Cu(II) 2p1/2,sat |  | Cu(0) | Cu(II) |
| 1 L ‒ cathode | 929.7 | 931.9 | --- | 949.9 | 951.9 | --- |  | 75 | 25 |
| 1 L ‒ anode | --- | 931.3 | 936.7 | --- | 951.8 | 956.4 |  | 0 | 100 |
| 40 L ‒ cathode | 929.0 | 931.9 | --- | 950.8 | 952.1 | --- |  | 60 | 40 |
| 40 L ‒ anode | --- | 929.0 | 932.7 | --- | 949.6 | 953.3 |  | 0 | 100 |
|  | Binding energy (eV) |  |  |  |  |  |  | Composite (%) |  |
|  | Ni(0) 2p3/2 | Ni(II) 2p3/2 | Ni(II) 2p3/2,sat | Ni(0) 2p1/2 | Ni(II) 2p1/2 | Ni(II) 2p1/2,sat |  | Ni(0) | Ni(II) |
| 1 L ‒ cathode | 853.6 | 854.8 | 857.0 | 872.8 | 876.4 | --- |  | 74 | 26 |
| 1 L ‒ anode | --- | 853.2 | 859.4 | --- | 873.0 | 878.3 |  | 0 | 100 |
| 40 L ‒ cathode | 853.2 | 855.2 | 860.3 | 871.7 | 874.1 | 880.3 |  | 60 | 40 |
| 40 L ‒ anode | --- | 853.8 | 860.0 | --- | 871.6 | 876.4 |  | 0 | 100 |
|  | Binding energy (eV) |  |  |  |  |  |  | Composite (%) |  |
|  | Zn(0) 2p3/2 | Zn(II) 2p3/2 | Zn(0) 2p1/2 | Zn(II) 2p1/2 |  |  |  | Zn(0) | Zn(II) |
| 1 L ‒ cathode | 1019.0 | 1021.5 | 1043.6 | 1045.2 |  |  |  | 52 | 48 |
| 1 L ‒ anode | --- | 1020.9 | --- | 1044.0 |  |  |  | 0 | 100 |
| 40 L ‒ cathode | 1019.9 | 1021.1 | 1042.3 | 1044.0 |  |  |  | 47 | 53 |
| 40 L ‒ anode | --- | 1019.5 | --- | 1043.1 |  |  |  | 0 | 100 |
|  | Binding energy (eV) |  |  |  |  |  |  | Composite (%) |  |
|  | Cr(VI) 2p3/2 | Cr(III) 2p3/2 | Cr(VI) 2p3/2,sat | Cr(VI) 2p1/2 | Cr(III) 2p1/2 | Cr(VI) 2p1/2,sat |  | Cr(III) | Cr(VI) |
| 1 L ‒ cathode | 568.9 | --- | 574.8 | 585.1 | --- | 588.0 |  | 100 | 0 |
| 1 L ‒ anode | 574.8 | 576.7 | --- | 583.9 | --- | 585.9 |  | 51 | 49 |
| 40 L ‒ cathode | 568.7 | --- | 574.3 | 584.1 | --- | 587.4 |  | 100 | 0 |
| 40 L ‒ anode | 574.8 | 576.8 | --- | 584.5 | --- | 585.9 |  | 58 | 42 |

Table S4 Operational taxonomic units (OTUs), and species richness and diversity estimates of the anodes and the cathodes of 1 L or 40 L reactors fed with low or high strengths etching terminal wastewater

| Sample | Number | OTUs | ACE | Shannon | Chao | Simpson | Shannoneven | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1HAno | 25957 | 317 | 331 | 4.12 | 332 | 0.04 | 0.71 | 0.999 |
| 1HCat | 21780 | 395 | 477 | 4.13 | 503 | 0.05 | 0.69 | 0.996 |
| 1LAno | 24520 | 341 | 385 | 3.84 | 393 | 0.06 | 0.66 | 0.998 |
| 1LCat | 24910 | 421 | 462 | 4.22 | 471 | 0.05 | 0.70 | 0.997 |
| 40HAno | 24937 | 298 | 310 | 3.94 | 310 | 0.05 | 0.69 | 0.999 |
| 40HCat | 59764 | 638 | 640 | 4.73 | 641 | 0.03 | 0.73 | 1.000 |
| 40LAno | 62378 | 340 | 364 | 3.63 | 366 | 0.09 | 0.62 | 0.999 |
| 40LCat | 20285 | 274 | 302 | 3.95 | 310 | 0.04 | 0.70 | 0.998 |
| Total | 264531 | 3024 |  |  |  |  |  |  |